| OVERVIEW | K2 Quick Reference Card | Display Filter # & BW: Press Xfil + AGC | AGC ON/OFF: Press PRE/ATT + AGC |
| --- | --- | --- | --- |
|  | Top Label=Tap |  |  |
|  | Bottom Label=Hold ½ second | Restore factory default: (RESET ALL K2 FUNCTIONS) Turn off K2, hold 4,5,6 and turn back on. |  |
|  | Display: Volts / Amps |  |  |
|  | Play/Rec Keyer: Press MSG; Press 0-9 | Direct Frequency Entry: Hold BAND+ + BAND-, Enter 4 or 5 digits |  |
|  | Auto repeat: Tap MSG, Hold 0-8 | Set Power to 3 watts or less, before using ATU |  |
|  | PTT/SPEECH: Hold VOX ("L" or "U" flashes when in SSB VOX mode) | Scan - Setup (single-band scan only) Store VFOA (lower) & VFOB (upper) for band edges of interest. Select operating mode: preamp / attenuator, tuning rate. Press STORE; Press 0-9 |  |
|  | Lock: decimal flashes |  |  |
|  | Test/Operate: Hold VOX, CW mode C flashes | Scan (initiate): Hold 0-9 when STORE or RCL a Memory. |  |
|  | Split frequency: A or B flashes when on | Hold LEVEL once to set noise blanker threshold Low, Hold LEVEL again to set noise blanker threshold High |  |
|  | RIT / XIT: 2 RIT/XIT Ranges +/- 0.6, +/-1.2 KHz. flashes when RIT2 on | Tap NB once to turn on nb1, Tap NB again to turn on nb2, Tap NB again to turn off noise blanker |  |

| PROCEDURES | Set BFO/FIL: FCTR in TP2, Band between 160-17M, Select CW, CAL FIL, Hold Edit. Band+=BW, Band-=BFO. Use Display to see BFO frequencies and set as in table below; Xfil / Mode to switch. [*BF1t: "t" = SSB TX BFO. Change with caution.] | Linearize VCO: FCTR in TP1. Select VFO frequency, CAL PLL, Hold edit. 1800.1, 3500.1, 7000.1, 10000.1, 14000.1, 18000.1, 21000.1, 24800.1, 28000.1 |  |
| --- | --- | --- | --- |
|  | S LO: RF Gain CW, turn VFO Knob CW until left segment turns off. About 180. | Program PF1/PF2: Most useful PF1/PF2 settings: RIT, ATU, SCAN, SSBC. |  |
|  | S HI: RF gain CCW, turn VFO Knob CCW unit right segment just turns on. About 22 | Peak bandpass max signal, then power out: (in order) 80: L3/L4; 40: L1/L2; 30: L8/L9; 20: C21/C23; 17: C32/C34; 10; L12/L13, 12: C44/C46 |  |
|  | Power Save: LCD Day, Grph off, Opt Battery, Preamp off, Use headphones |  |  |
|  | Frequency calibration: FCTR and external counter on TP1, select 28 MHz, match counters using CTRL-C22. [You must re-run CAL PLL (on all bands) and CAL FIL (tweak all BFOs) before you'll get any benefit out of the C22 change! ] |  | VCO Align: Frequency set to 4000.1, DMM to left side of R30). Adjust L30 for 6.0 Volts |

| Mode | FL1 | BF1t * | FL2 | BF2 | FL3 | BF3 | FL4 | BF4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CW/REV (4913.6/16) = (4913.6/4916.0) | OP1 | 4913.6/16.0 | 0.7 | 4913.1/14.3 | 0.4 | 4913.1/14.3 | 0.1 | 4913.0/14.2 |
| LSB/USB (4913.5/16.2) = (4913.6/4916.2) | OP1 | 4913.5/16.2 | OP1 | 4913.5/16.1 | 1.8 | 4913.0/15.5 | 0.7 | 4912.7/14.7 |

| ### MENU COMMANDS |  | Select with main knob, BAND+ or BAND-, Hold EDIT to change: |  |
| --- | --- | --- | --- |
| ST L / ST P: Set Side Tone [L]evel / [P]itch |  |  |  |
| t-r: QSK delay. 0.02 Default | rPt: CW message Repeat intrvl |  | iAb: iambic A or B |
| inP: Paddle hand, normal, reverse | LCd: night-backlight on, day off |  | GrPH: dOt, bAr, OFF |
| CAL: Off, FCtr, FIL, PLL, CUR, S LO, S HI |  | PF1/2: Select User-programmed Function PF1/2 |  |
| SSbA: Audio, SSbC: Speech Compressor | rAnt: Aux Rec Ant switch OFF, ON per Band |  | OPt: Perf, Battery |
| ATU: Automatic Tuning Unit |  | L0-L8 / C0-C8: Test individual ATU element |  |
| CALn: Used for calibration of C55 (null). L/C set to 0; network set to Cin. |  | AUTO: Normal auto-tune; use with most antennas |  |
|  |  | POUT: Forward/Reverse power display |  |
| CALP: Used for calibration of R1. L/C set to 0; network set to Cin. |  | ALT: Alternate auto-tune; use with short whips, etc. (much longer tune times) |  |
| CALS: Used to check SWR calibration. L/C set to 0; network set to Cin. |  | Lxx.x / Cx.xx: Ind(uH)/Cap(nF) in use*** |  |
|  |  | x.x-1: SWR from most recent tune-up*** |  |
| INFO CODES: 010 Battery Voltage too low, 80 IOC Problem, 81 AuxBus Problem, 90/91 EEPROM write test failed, 201 EEPROM initialized, 230 BFO not connected to Freq Cntr, 231 VCO not connected to Frequency Counter, 235 PLL Ref Oscillator range error, |  |  |  |

##### ***(stored per-antenna, per-band)

##### BAND LISTING (Band, Band Segments, QRP Calling Frequencies-Novice, Europe, PSK31)

|  |  |  | CW |  |  |  |  |  | SSB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 160 | 1.8 |  | 1.81 |  | 1.838 |  |  | 1.843 | 1.91 | 2.0 |
| 80/75 | 3.5 | 3.56 | 3.579 | 3.58 | 3.71 | 3.75 |  | 3.69 | 3.985 | 4.0 |
| 40 | 7.0 | 7.035 7.04 | 7.080 7.035 | 7.03 7.06 | 7.110 7.112 | 7.15 |  | 7.09 | 7.285 | 7.3 |
| 30 | 10.1 |  | 10.106 |  | 10.116 |  |  | 10.140 |  | 10.15 |
| 20 | 14.0 | 14.06 |  |  | 14.070 | 14.15 |  |  | 14.285 | 14.35 |
| 17 | 18.068 |  | 18.096 |  | 18.100 | 18.11 |  |  | 18.13 | 18.168 |
| 15 | 21.0 | 21.06 | 21.110 |  | 21.080 | 21.2 |  | 21.285 | 21.385 | 21.45 |
| 12 | 24.89 |  | 24.906 |  | 24.920 | 24.93 |  |  | 24.95 | 24.990 |
| 10 | 28.0 | 28.06 | 28.110 |  | 28.120 | 28.3 | 28.36 | 28.385 | 28.885 | 29.7 |
| K2 |  |  |  |  |  |  |  |  | 28.715 |  |